How to Flash Voyager ROW

- Download and extract the phone SW binary

- Set carrier uTags and flash the phone SW

  - Put phone in fastboot mode (Volume Down + Power key) and connect to PC using USB-TypeC cable.

  - Open a Windows, Linux or Mac shell. Make sure you are in the correct folder.

  - Run the below sequence of shell commands:

Erase Userdata:

fastboot erase cache fastboot erase userdata fastboot erase frp fastboot erase modemst1 fastboot erase modemst2

Select channel ID:

Windows, use command:

programutags.bat

Linux/Mac, use command:

./programutags.sh

Choose your channel ID by navigating through the programutags menu. Select the ro.carrier.

In case the carrier is not present in programutags menu, you can also config it via the following fastboot command:

fastboot oem config carrier <ro.carrier>

- https://idart.mot.com/browse/IKSWP-109101 raised for providing programutags fixes.

Flash phone SW:

Windows, use command:

flashall.bat

Linux/Mac, use command:

./flashall.sh

Finally, wait for the phone to boot up.